MatrixPortal M4 Text Protocol Firmware

--------------------------------------

Listens on Serial1 for simple '|' delimited commands and drives

a 64x32 HUB75 RGB matrix using the Adafruit_Protomatter library.

Supported commands (newline-terminated):

C

Clear screen, stop scrolling, reset line states.

COLOR|value

Set global text color. 'value' can be:

- #RRGGBB (hex)

- RED, GREEN, BLUE, WHITE, YELLOW, CYAN, MAGENTA

TEXT|row|text

Draw plain text on row 0..3 using the current color.

BOLD|row|text

Draw "bold" text on row 0..3 (text drawn twice with 1px offset).

FLASH|row|hz|text

Flash text on row 0..3.

hz = flash frequency in Hz (integer). If missing or invalid, defaults to 1.

SCROLLH|dir|text

Horizontal scrolling text, full-screen.

dir = LEFT or RIGHT (default LEFT if missing/invalid).

SCROLLV|dir|text

Vertical scrolling text, full-screen.

dir = UP or DOWN (default UP if missing/invalid).

Protocol details:

- Fields are separated by '|'.

- Newline '\n' terminates a command.

- Everything after the last '|' is treated as raw text (no escaping).

Hardware link:

- Connect sender TX -> MatrixPortal RX (Serial1)

- Connect sender GND -> MatrixPortal GND

- Optional: MatrixPortal TX -> sender RX (for debugging/ACKs)

Examples:

C\n

COLOR|WHITE\n

TEXT|0|Feather M0\n

TEXT|1|Matrix Demo\n

COLOR|GREEN\n

SCROLLH|LEFT|Hello from Feather M0!\n

COLOR|#FF00FF\n

SCROLLV|UP|LoRa-ready sender\n

C\n

COLOR|YELLOW\n

FLASH|0|2|Alert\n

TEXT|1|Radio OK\n

LoRa Over the Air Text String Example

$BSTXT,<station_id_hex>,<payload>*<CS>

Example:

$BSTXT,0A,C\\nCOLOR|WHITE\\nTEXT|0|Next Station\\nTEXT|1|Bos\\\\Camb\\n*32

Where:

- Talker ID: "BS"  (Base Station)

- Type: "TXT"

- station_id_hex: 2-digit hex (e.g., "0A")

- payload: text protocol for MatrixPortal M4, with escaping:

\n  -> newline separator

\\  -> literal '\'

- The Feather M0 or MKR WAN 1310:

* Verifies checksum (XOR of all chars between '$' and '*')

* Checks station ID against STATION_ID

* Unescapes \n and \\

* Forwards decoded text to MatrixPortal via Serial1

MatrixPortal M4 still expects:

C

COLOR|WHITE

TEXT|0|Next Station

TEXT|1|Bos\Camb

...with real '\n' between commands.